Heterogeneous OS Benchmarking for Real-Time Edge Control on Asymmetric Hardware

Abstract

Industrial Cyber-Physical Systems (CPS) are undergoing a critical evolution from open-loop Predictive Maintenance (PdM) - where algorithms merely alert human operators to upcoming faults to closed-loop Prescriptive Maintenance (PsM), where embedded controllers autonomously intervene to throttle motor speeds and prevent catastrophic failures within millisecond deadlines.

This operational shift exposes a fundamental operating system dilemma. While an idle Linux system can comfortably execute signal processing and neural network inference within average timing budgets, General-Purpose Operating Systems (GPOS) are architecturally unsuited for hard real-time guarantees. Under unpredictable background loads, memory pressure, or I/O contention, Linux's Completely Fair Scheduler (CFS) and virtual memory demand paging introduce involuntary context switching and multi-millisecond page fault stalls. In safety-critical automation, where worst-case latency matters far more than average throughput, a single latency spike can lead to mechanical destruction.

To resolve this conflict, we investigate a Heterogeneous Dual-OS Architecture on the Arduino UNO Q, an Asymmetric Multi-Processing (AMP) platform combining a quad-core Linux microprocessor (Qualcomm QRB2210 Cortex-A53) with a deterministic real-time microcontroller (STM32U585 Cortex-M33). By cleanly decoupling computational responsibilities—offloading heavy vibration analytics to Linux while isolating time-critical actuation onto Zephyr RTOS via a bounded Inter-Processor Communication (IPC) bridge—the system achieves both computational power and fault-isolated timing determinism.

The objective of this project is to quantitatively evaluate this heterogeneous approach against monolithic Linux configurations (standard CFS and privileged SCHED_FIFO) across 10,000 continuous closed-loop cycles under escalating synthetic stress (stress-ng). By capturing kernel-level telemetry (getrusage context switches, minor/major page faults, and hardware DWT cycle jitter), we correlate micro-architectural OS stalls with macroscopic Deadline Miss Ratios (DMR) and Worst-Case Execution Time (WCET). Our empirical findings aim to provide concrete architectural guidelines for designing resilient, mixed-criticality industrial edge controllers.